## Clean room instructions

This document serves to establish guidelines and procedures for the proper utilization, maintenance, and operation of the clean room to ensure a controlled environment with minimal contamination. These guidelines apply to all personnel who access and work within the clean room. It covers entry and exit procedures, proper attire, equipment handling, and environmental controls.

Clean room at HEP lab can be accessed from the gowning room, and these are maintained at two different air pressures. Clean room is maintained at over pressure, and gowning room is at lower pressure.

- Entry instructions:

  - Only authorized personnel with UNL access card may enter the clean room.

  - Check or turn on the vacuum pumps if you want to utilize them in the clean room. Vacuum pumps are connected with the gantry and the wire bonding machine through the wall pipe and for which the control is setup in the general area of the lab.

  - Before entry, personnel must have an appropriate clean room attire, including but not limited to:

    - Hairnets

    - Face masks

    - Gloves

    - Cleanroom suits (coveralls)

    - Shoe covers or designated cleanroom footwear

  - Use the air shower (if available) before entering the clean room. For HEP lab clean room, we don’t have such a requirement.

  - Hands must be sanitized before wearing gloves.

  - Gowning door must be closed before opening the clean room door.

- Code of conduct inside the clean room

  - Wait for five minutes before starting work.

  - This will allow the particles to be taken out by the HEPA filters from the clean room.

  - Minimize movement and talking to reduce contamination.

  - Avoid unnecessary touching surfaces.

  - Do not bring personal items such as cell phones, food, or beverages inside.

  - Use proper guidelines to handle the tools and equipment.

- Equipment handling:

  - Wipe down all equipment with an appropriate disinfectant before use.

  - Ensure tools are stored in designated areas when not in use, and the areas are properly marked or labeled.

  - Follow the clean room standards to minimize the equipment contamination.

- Cleaning and maintenance:

  - Follow the scheduled cleaning protocol using approved cleaning agents.

  - Clean all work surfaces before and after each use.

  - Dispose of waste materials in designated bins.

  - Replace the sticky mats weekly or if you feel that they need to be replaced earlier.

  - Regularly monitor and record environmental parameters as required. For this, we have hand handled particle counter.

  - Maintain proper temperature, humidity, and air filtration settings as specified. For this check regularly with the building facility manager.

- Exit instructions:

  - Remove clean room attire in the designated (gowning) changing area.

  - Properly dispose of single-use items such as gloves and masks.

  - Exit through the designated exit door without re-entering unless necessary.